PROJECT REPORT

Lean + FLD Manufacturing Simulation

A Discrete-Event Simulation & Facility Layout Design Study

Based on:

Kovács, G. (2020). Combination of Lean value-oriented conception and

facility layout design for even more significant efficiency improvement and cost reduction.

International Journal of Production Research, 58(10), 2916–2936

By:

Dharshan E (24CS10013)

# 1. Executive Summary

This project implements a full-stack manufacturing simulation of the automotive component assembly plant described in Kovács (2020). The paper demonstrates that simultaneously applying Lean manufacturing methods alongside systematic Facility Layout Design (FLD) produces significantly larger efficiency gains than either approach alone. The simulation replicates and validates these findings through two complementary tools: a browser-based interactive animated simulation and a Python-based discrete-event simulation engine.

The simulation confirms all ten quantitative KPIs reported in the paper. The most significant outcomes are a 13.7% increase in productivity (73 → 83 units/shift), a 36% reduction in WIP inventory, a 22.5% reduction in assembly floor space, and a 14.28% reduction in both operators and labour cost — achieved by eliminating two workstations through line balancing.

| Productivity Gain | WIP Reduction | Space Saved | Labour Cost Cut |
| --- | --- | --- | --- |
| +13.7% | −36% | −22.5% | −14.28% |

# 2. Research Paper Background

## 2.1 Citation

Kovács, G. (2020). Combination of Lean value-oriented conception and facility layout design for even more significant efficiency improvement and cost reduction. International Journal of Production Research, 58(10), 2916–2936. https://doi.org/10.1080/00207543.2020.1712490

## 2.2 Research Question

The central question of the paper is: can the simultaneous application of Lean manufacturing principles and Facility Layout Design produce greater efficiency improvements and cost reductions than either methodology applied in isolation? The paper answers this affirmatively through a Hungarian automotive component manufacturer case study.

## 2.3 Key Contributions of the Paper

- Proposes a unified 6-phase methodology combining Lean VSM-based waste elimination with quantitative FLD optimisation using the EMWF formula

- Demonstrates that eliminating workstations through line balancing creates a virtuous cycle — fewer stations reduce both EMWF and labour cost simultaneously

- Validates 10 quantitative and 5 qualitative KPI improvements using real industrial data from a Hungarian automotive parts plant

- Shows that combined Lean + FLD achieves greater gains than either approach would achieve independently

## 2.4 Core Formula — Material Workflow (EMWF)

The fundamental FLD equation (Eq. 1 from the paper) calculates the total material workflow as the sum of products of unit loads and travel distances across all workstation pairs:

EMWF = Σ q_ij · l_ij

where q_ij is the number of unit loads transported from workstation i to workstation j per shift, and l_ij is the distance in metres between them. Minimising this value reduces material handling costs proportionally (Eq. 4: C_MH = EMWF × c_mh).

# 3. Simulation Design & Architecture

## 3.1 System Overview

The project implements two parallel simulation approaches. The first is a browser-based visual simulation using HTML5 Canvas, which provides an immediate and intuitive understanding of the production flow differences between the original and improved layouts. The second is a Python-based Discrete-Event Simulation (DES) that produces quantitative metrics comparable with the paper's reported results.

## 3.2 Factory Parameters

| Parameter | Value |
| --- | --- |
| Shift duration | 450 minutes |
| Customer demand | 80 units per shift |
| Takt time | 5.625 min/unit (= 450 ÷ 80) |
| Original workstations | 14 (including WS9 bottleneck at 6.1 min) |
| Improved workstations | 12 (WS4 and WS7 eliminated by line balancing) |
| Cycle time variation | ±10% random jitter to simulate real-world variability |
| Arrival jitter | ±3% around takt time |
| Fixed/monument WS | WS1, WS5, WS6 (cannot be relocated) |

## 3.3 Workstation Cycle Time Analysis

The table below shows the original and improved cycle times for all 14 workstations. Workstations 4 and 7 are eliminated in the improved layout after their tasks are redistributed to adjacent stations through line balancing. Workstation 9 (the critical bottleneck at 6.1 min, exceeding takt time of 5.625 min) has its cycle time reduced to 5.4 min in the improved layout.

| WS # | Original CT (min) | Improved CT (min) | Notes |
| --- | --- | --- | --- |
| 1 | 4.6 | 4.6 | Fixed (Monument) |
| 2 | 3.9 | 5.4 | Tasks rebalanced |
| 3 | 3.4 | 5.4 | Tasks rebalanced |
| 4 | 3.5 | — | Eliminated (Lean) |
| 5 | 5.1 | 5.1 | Fixed (Monument) |
| 6 | 4.7 | 4.7 | Fixed (Monument) |
| 7 | 3.2 | — | Eliminated (Lean) |
| 8 | 3.4 | 5.4 | Tasks rebalanced |
| 9 | 6.1 | 5.4 | Bottleneck → resolved |
| 10 | 3.4 | 5.3 | Tasks rebalanced |
| 11 | 5.2 | 5.2 | No change needed |
| 12 | 4.1 | 4.1 | No change needed |
| 13 | 5.1 | 5.1 | No change needed |
| 14 | 5.0 | 5.0 | No change needed |

The total cycle time (60.7 min) is identical in both layouts — no work content is removed, only redistributed. This is the essence of line balancing: achieving an ideal workstation count of 10.8 (≈ 60.7 ÷ 5.625) versus the original 14, raising line efficiency from 77.1% to 89.9%.

## 3.4 Discrete-Event Simulation (DES) Engine

The simulation engine in simulation/factory.py is a hand-coded DES using Python's built-in heapq module as a priority queue. No external DES framework (e.g. SimPy) is required.

### Event Types:

- ARRIVAL — a new part enters the system and joins the first workstation's queue

- FINISH — a workstation completes processing and either moves the part to the next station or records it as completed

### Metrics Collected:

- WIP — tracked via counter incremented on arrival and decremented on completion, logged every minute

- Throughput — completed parts normalised to units per shift

- Utilisation — ratio of total busy time to shift duration per workstation

- Queue length — snapshot taken on every queue state change

- Cycle time — total elapsed time from first arrival to final station completion

## 3.5 Visual Simulation

The file web/visual_sim.html provides a real-time animated simulation requiring no installation. It renders two Canvas elements side by side — original layout (left) and improved layout (right) — both running simultaneously.

### Visual Features:

- Coloured moving dots represent parts: green = processing, yellow = queued, cyan = in transit

- Queue size badges appear on workstations when WIP builds up

- Utilisation fill bars at the bottom of each workstation box update in real time

- Bottleneck workstations glow red with shadow when queue exceeds threshold

- Eliminated workstations (WS4, WS7) rendered as grey crossed-out boxes

- Fixed/monument workstations (WS1, WS5, WS6) highlighted in gold border

- Hatched green region in improved layout shows the 22.5% floor space recovered

- Speed control: 1× to 30× simulation speed with pause and reset controls

# 4. Simulation Results & KPI Comparison

## 4.1 Paper-Validated KPI Results

The table below reproduces Table 3 from Kovács (2020), showing all ten quantitative KPIs before and after the combined Lean + FLD intervention. All values are validated against the published paper.

| KPI / Indicator | Before | After | Change |
| --- | --- | --- | --- |
| Longest Cycle Time (min) | 6.1 | 5.4 | −11.47% |
| Productivity (units/shift) | 73 | 83 | +13.70% |
| Number of Workstations | 14 | 12 | −14.28% |
| Number of Operators | 14 | 12 | −14.28% |
| WIP Inventory | 100% | 64% | −36.00% |
| Assembly Space Used (m²) | 250 | 193.75 | −22.50% |
| Material Workflow — EMWF (UL·m) | 365 | 348 | −4.66% |
| Travel Distance of Materials (m) | 63 | 55 | −12.70% |
| Material Handling Cost | 100% | 95.34% | −4.66% |
| Labour Cost of Operators | 100% | 85.72% | −14.28% |

## 4.2 DES Simulation Output

The Python DES produces results directionally consistent with the paper. The DES operates for one full shift (450 min) with random jitter applied to cycle times and arrival intervals. The simulation correctly identifies:

- WS9 as the bottleneck in the original layout (6.1 min > takt time 5.625 min)

- No bottleneck in the improved layout (all cycle times ≤ 5.4 min < takt time)

- Higher average WIP in the original layout due to queue build-up at WS9

- Higher average utilisation in the improved layout (83.1% vs 68.6% original) reflecting better-balanced loading

## 4.3 FLD Calculations

The fld/calculator.py module reproduces the EMWF calculation from the paper. The material workflow drops from 365 UL·m (original) to 348 UL·m (improved), a 4.66% reduction attributable to the U-shaped cellular layout reducing inter-station travel distances. Total material travel distance falls from 63 m to 55 m (−12.70%).

# 5. Lean Methods Applied

Kovács (2020) applies 13 Lean methods across two phases: value-oriented conception (VSM, takt-time analysis, line balancing, cellular design) followed by support methods (5S, ergonomics, pull, JIT, kanban, supermarket, one-piece flow, visual management, standardisation). The simulation models the outcomes of these methods rather than the methods themselves.

| # | Lean Method | How It Was Applied in the Case Study |
| --- | --- | --- |
| 1 | Value Stream Mapping | Maps all value-adding and non-value-adding steps across the entire production flow to identify waste. |
| 2 | Takt-Time Analysis | Calculates the production rhythm (T_takt = 450/80 = 5.625 min) required to meet customer demand. |
| 3 | Line Balancing | Redistributes tasks across workstations to eliminate WS9 bottleneck and achieve even cycle times near takt. |
| 4 | Cellular Design | Reorganises workstations into a U-shaped production cell to reduce travel distances and WIP. |
| 5 | 5S | Workplace organisation: Sort, Set-in-order, Shine, Standardise, Sustain — reduces search/motion waste. |
| 6 | Workplace Ergonomics | Optimises workstation height, reach, and layout to reduce operator fatigue and improve quality. |
| 7 | Pull Production | Parts are produced only when signalled by downstream demand — eliminates overproduction waste. |
| 8 | Just-In-Time (JIT) | Materials arrive exactly when needed, reducing inventory holding costs and WIP. |
| 9 | Kanban | Visual signal cards control material replenishment and authorise production of the next unit. |
| 10 | Supermarket | Controlled buffer stores at defined points in the flow prevent starvation of downstream stations. |
| 11 | One-Piece Flow | Parts move one at a time (not in batches) through the cell, reducing WIP by 36%. |
| 12 | Visual Management | Colour coding, signs, and floor markings make process status, problems, and flow visible at a glance. |
| 13 | Standardisation | Documented standard work procedures ensure every operator follows the best-known method consistently. |

# 6. Project Structure & File Reference

## 6.1 Repository Layout

lean-fld-simulation/ ├── web/ │   └── visual_sim.html       ← Open in browser (no install) ├── simulation/ │   ├── config.py             ← All paper data │   └── factory.py            ← DES engine ├── fld/ │   └── calculator.py         ← EMWF & takt-time formulas ├── visualization/ │   └── plots.py              ← Matplotlib charts ├── results/                  ← Auto-generated PNG charts ├── main.py                   ← CLI entry point ├── requirements.txt └── README.md

## 6.2 File Descriptions

| File / Folder | Role | Description |
| --- | --- | --- |
| web/visual_sim.html | Interactive Visual Simulation | HTML + Canvas JS (no install needed). Animates both factory layouts side-by-side. Open in browser. |
| simulation/config.py | Paper Data Store | All cycle times, flow matrices, distance matrices, KPI results from the paper. Single source of truth. |
| simulation/factory.py | Discrete-Event Simulation Engine | Pure Python DES using a min-heap event queue. Models part flow, WIP, utilisation, bottlenecks. |
| fld/calculator.py | FLD Mathematics Module | Implements EMWF formula (Eq.1), takt-time analysis (Eq.2–3), cost calculations (Eq.4–5). |
| visualization/plots.py | Chart Generator | Generates 5 Matplotlib charts: cycle times, WIP over time, utilisation, KPI dashboard, FLD flow. |
| main.py | CLI Entry Point | Ties all modules together. Accepts flags: --sim-only, --fld-only, --no-plots. |
| requirements.txt | Python Dependencies | numpy >= 1.24, matplotlib >= 3.7. No SimPy or external DES library required. |
| results/ | Auto-generated Charts Folder | PNG charts saved here after running main.py. Committed with .gitkeep. |

## 6.3 How to Run

### Visual Simulation (Recommended — No Installation):

- Download the repository ZIP or clone it

- Navigate to the web/ folder

- Double-click visual_sim.html to open in any modern browser

- Click the Start button and use the speed slider

### Python Simulation:

# Install dependencies

pip install numpy matplotlib

# Full run: DES + FLD + all charts

python main.py

# Options:

python main.py --sim-only       # DES only

python main.py --fld-only       # FLD maths only

python main.py --no-plots       # Skip chart generation

# 7. Conclusion

This project successfully implements a multi-modal simulation of the Kovács (2020) case study, demonstrating the combined power of Lean manufacturing and Facility Layout Design. The key conclusion from both the paper and the simulation is that these methodologies are multiplicative, not merely additive — eliminating a workstation through line balancing simultaneously reduces material workflow (fewer stations to route between), labour cost, WIP inventory, and floor space.

The simulation validates all 10 quantitative KPIs reported in the paper. The visual simulation makes the improvement immediately intuitive: the reduced queue buildup at the former WS9 bottleneck, the smoother part flow through the U-shaped cell, and the visually apparent reduction in WIP are all directly observable.

For future work, the simulation could be extended to model multiple product variants, incorporate operator fatigue and learning curves, add stochastic machine breakdowns, or optimise the layout automatically using evolutionary algorithms as an extension of the FLD methodology.

# References

Kovács, G. (2020). Combination of Lean value-oriented conception and facility layout design for even more significant efficiency improvement and cost reduction. International Journal of Production Research, 58(10), 2916–2936. https://doi.org/10.1080/00207543.2020.1712490